GENCORN BIOENGINEERING CO., LTD.             THE INSPECTION REPORT OF NARINGIN DIHYDROCHALCONE
CERTIFICATE OF ANALYSIS

Product Name: Naringin Dihydrochalcone             Specification: 98%
Batch No.:   250207                       Batch Quantity: 500kg
Manufacture Date: Feb.12, 2025                 Report Date: Feb.20, 2025

Expiry Date: Feb.11, 2027                    CAS#: 18916-17-1
	Item
	Specification
	Results

	Characteristics
	White or yellowish odorless powder
	  White odorless powder

	Solubility
	Practically insoluble in water, freely soluble in DMSO
	Conforms

	Identification

	RT of Main Peak 

(HPLC)
	The retention time corresponds to Reference Standard 
	Conforms

	IR Absorption
	Exhibits maxima at the same wavelengths to the Reference Standard
	Conforms

	Test

	Loss on Drying
	≤5.0%
	4.4%

	Heavy Metals
	 ≤10ppm
	Conforms

	As
	≤2ppm
	Conforms

	Residue on Ignition
	≤0.2%
	<0.1%

	Microbial Limit

	Total Plate Count
	   ≤1000cfu/g
	207cfu/g

	Yeast and Moulds
	 ≤100cfu/g
	＜10cfu/g

	Escherichia coli
	Negative
	Conforms

	Assay（the dried basis）
	≥98.0%
	98.7%

	Conclusion
	The test results conform To Specification.


Analyst: 06                                   QC Supervisor: 01
